Dwin Display Communication & Setup Guide

Part 1: Display & Main Source Code Communication Interface

The communication between the Dwin Touch Screen HMI and the STM32F100RB Application Layer follows a decoupled, event-driven architecture over a physical RS232 bus. The system abstracts this communication using a packet-based register mapping system.

1. Data Link Parameters

- Physical Layer: RS232 (Voltage-shifted to TTL for STM32 UART peripheral access)

- Baud Rate: 115200 bps

- Data Framing: 8 Data bits, 1 Stop bit, No Parity

2. Protocol Frame Format

- Every data exchange between the STM32 firmware and the DWIN screen follows the standardized DWIN framing architecture:

0x5A,0xA5,0x07,0x82,0x20,0x20,0x00,0x00

- Frame Header: Fixed 0x5A 0xA5 to signal the start of a valid transmission frame.

- Command Types: Typically, 0x82 (Write to Display Variable Space) and 0x83 (Read from Display Variable Space).

- VP Register Mapping: The core connection mechanism. RAM variables on the display are assigned a specific Variable Peripheral (VP) hexadecimal address.

3. Software Integration & Interaction Flow

Instead of hardcoded visual linkages, the screen and the STM32 firmware interact purely via state-based memory updates:

- HMI Event (Input): When a user triggers an asset on the display (e.g., hitting the "Treat/Standby Mode" button or checking the "Delivery Device" status), the Dwin hardware automatically writes that state value to its internal VP register and transmits the corresponding 0x83 read-response packet down the RS232 line.

- STM32 Ingestion: The STM32 UART peripheral captures the raw incoming stream using Rx Interrupt Buffer.

- Command Parsing: The Embedded C Application Layer parses the frame packet, filters for the expected 0x5A 0xA5 header, validates the data length, and extracts the target VP address.

- State Execution: The parsed value is passed into the system state engine (referenced in the UML Behavioural Diagrams), triggering low-level HAL Driver actions (such as enabling the Laser Driver GPIO pin).

- HMI Update (Output): When the system status changes (e.g., an error trigger from the M2 or Acc_Key peripherals), the STM32 sends a 0x82 write frame to update the corresponding VP address on the screen, updating the graphics in real-time.

Part 2: Development Environment Setup Guidelines

1. Prerequisite Hardware Components

- PC/Laptop with an available USB port.

- Dedicated USB-to-RS232/Serial Converter board or industrial cable.

- Target device prototype unit (containing the STM32F100RB control board and Dwin LCD module).

2. Software Tools Installation Requirements

- Display GUI Environment: DGUS TOOL software (Used to open the provided Dwin project file, modify image assets, and configure screen register grids)

- Firmware IDE: STMicroelectronics STM32CubeIDE 1.8.0 configured with the standard ARM GCC Compiler toolchain.

- Flashing & Debug Utilities: STM32 ST Link – Utility alongside an authentic ST-LINK V2/V3 hardware debugger interface.

3. Step-by-Step Initial Environment Build Setup

- Dwin HMI Verification: Open the DGUS Tool software. Load the Dwin project directory. If visual layouts or VP register allocations are modified, run the "Generate" routine to rebuild the target flash binaries (typically outputting files like 13.bin, 14.bin, etc., into the DWIN_SET compiler folder).

- STM32 Workspace Configuration: Open STM32CubeIDE and choose your local path workspace. Import the provided Embedded C application source repository.

- Code Compilation: Execute a clean project build (Ctrl + B). Verify that the project compiles cleanly with 0 compilation errors. The compiler will generate the debug output file (.elf) and the deployment image binary (.bin).

- Device Provisioning: Connect the ST-LINK hardware debugger to the STM32 physical board development layout pins. Open ST-Link Utility, connect to the target microchip over the interface, and flash the compiled .bin asset directly into the system flash memory.

4. Dwin HMI Deployment & Flashing Procedure

The Dwin display kernel loads its graphical assets, fonts, and VP configurations via an external hardware interface. To flash the compiled display assets:

- Prepare the Media: Format a standard MicroSD card (strictly FAT32 allocation, with a cluster size of 4096 bytes, and a storage capacity under 16GB as required by the Dwin hardware architecture).

- Package the Assets: Compile the HMI project in the DGUS software. Locate the generated output files (e.g., .bin,. hzk, and .bmp files). Copy these files into a folder named exactly DWIN_SET at the root directory of the MicroSD card.

- Execute Hardware Flash: Turn off the power to the target device. Insert the prepared MicroSD card into the display's integrated SD slot. Power on the device.

- Verification: The display kernel will automatically detect the DWIN_SET folder, initialize a blue/orange flash status screen, and display the file download count. Once the screen indicates the download is 100% complete, turn off the power, remove the MicroSD card, and reboot the unit to run the updated firmware.

5. DWin HMI to STM32 Variable Peripheral (VP) Register Mapping

The interaction between the display UI and the STM32F100RB firmware is governed entirely by a shared memory register map. The table below outlines how key hardware peripherals map to specific VP Addresses using standard hexadecimal data framing.

| Peripheral / System Component | VP Address (Hex) | Data Type | Direction (Flow) | Architectural Description |
| --- | --- | --- | --- | --- |
| Set Power | 0x1000 | uint8_t | Display ➔ STM32 (Input) | The display updates the power value when user sets it; STM32 polls/reads this to set the desired power. |
| System Readiness Status | 0x8504 | uint8_t | STM32 ➔ Display (Output) | The STM32 evaluates the system integrity. It will issue the ready signal if all hardware connections are verified as OK, setting the operational system ready status. |
| Active Screen Page | 0x0C | uint8_t | Bidirectional (Control) | System mode tracking. Screen updates address on touch; STM32 can force page jumps during faults. |

Example Command Packet Stream Mechanics

To demonstrate how transparent this protocol interface is, a typical data packet command down the RS232 bus follows this basic hex array structure:

- Scenario: The hardware safety circuit detects that the Remote Interlock has been opened (Hazard state).

- Action: The STM32 sends a write frame to the display to change the visual warning state.

- Raw RS232 Packet Sent: 0x5A,0xA5,0x05,0x82,0x85,0x04,0x00,0x02

  - 5A A5: Frame Header (Start of packet synchronization)

  - 05: Data Length (5 bytes following this index)

  - 82: Command Type (Write to Dwin VP Memory space)

  - 85 04: Target Register Address (System Readiness Status VP Location)

  - 00 02: Target Value Payload (0x0002 = Ready State)